# Step 1: Prepare Wall

- Sound for Stud

- Measure mark and saw hole ( You piece of drywall is 1’ by 2’ you should center your hole in the top 1’ of the drywall, this way I can flip it over and use it again)

# Step 2: Pull Cable

- Measure Cable path (Give yourself 4’ to 5’ slack)

- Obtain & cut cable (calculate & tell me how much you want)

- Use cable snake to feed cable from socket to top of wall

- Use yellow line to feed through pipe A (This simulates going through a wall)

- Mark both ends so you can tell which cable is yours

## Step 3: Mount Electrical Box

- Run end of cable through electrical box

- Install box (The box should be firmly attached to the wall, if I can wiggle it and it comes out then it is not attached)

## Step 4: Mount Wall Plate

- Wire Socket using “B” Standard

- Mount socket to face plate

- Mount face plate to electrical box

## Step 5: Tie To Patch Panel

- Strip & prep end of cable

- Punch down to patch socket that corresponds to your station number (using “B” standard)

## Step 6: Test Cable

- Verify both ends are in place

- Use Cablemeter to test the line

- If it is bad fix it

## Step 7: Connect to network

- Use Patch cable on router rack to connect to Cisco switch

- Use Patch cable on bench to connect to computers on the bench

- Turn computer on and make it work

### Materials List

### Per Station

Cablemeter

Hole Saw

Patch Cables 2 per team

Punch Down tool

Screwdriver

#### Shared

Cable Snake

Stud Sounder